1-2.1 Motion System

The Motion System is driven by DC servomotors, which includes X and Y motor.  X motor, sits at right-hand side of X-rail, moves Lens Carriage left and right via X-axis belt, while Y motor drives X-rail into front and back motion with two Y-belts at either end of the rail simultaneously.

The motion system provides exact position or coordination to guide the laser beam engraving on the designated spot of work piece.  The laser beam come from laser tube reflected by mirrors and can be adjusted by prism mounts to pass through the focal lens that focuses the energy.

There are 3 reflective mirrors included in the motion system.  Mirror2 situates at the top left corner of the motion frame to receive the up-bounded laser and reflects it to Mirror3 that sits on the left side of X-rail.  After passing Mirror3, laser is reflected to Mirror4 that is fixed right on top of focal lens on the Lens Carriage.  Then, laser beam is guided downward through and focused after the focal lens.

The basic but very important maintenance of Motion System is a regular cleaning of rails and bearing’s surface.  The surfaces of rails, especially the X-rail, are easily accumulating dust and debris after running some engraving jobs, with wood in particular.  If they are not removed, the bearing on the Lens Carriage will roll with bumpy effect.  This bumpy effect is critical to laser, as any slight jarring impact will diver the beam path and affect the overall output quality.

The cleaning job can be carried out with a piece of dry and clean rug.  Wipe the surface of X-ail with rug, if necessary, dampen the rug with gentle cleaning solution.  To clean the bearing, press the rug against the U rollers or small rollers and shift the Lens Carriage by the other hand.   In general a weekly cleansing is recommended.  If you engrave or cut wood, glass or other debris prone materials, in a heavy-duty manner, a daily cleaning is necessary for retaining the output quality.

Do not lubricate the rails and bearings, as grease will attract more dust and dirt to their surfaces.